Firewall ICMP Rule Priority Test Procedure

firewall icmp move Command Verification

Date: 2026-04-03

1. Purpose

This procedure verifies that the firewall icmp move command correctly changes ICMP rule evaluation order. Rules are evaluated top-down (position 1 first). Moving a DROP rule above an ACCEPT rule should block traffic that was previously allowed.

2. Prerequisites

- Akita device with VPN connection established

- Admin access to the device CLI (scli)

- Local host (10.10.101.2) connected to the inside network interface

- VPN tunnel active to remote server (10.10.10.1)

- Static route on Local Host: 10.10.10.0/24 via 10.10.101.1 (VPN Client)

3. Network Configuration

The following diagram shows the test network topology:

| Local Host 10.10.101.2 | → | VPN Client (Akita) 10.10.101.1 192.168.100.133 | ===→ IPsec |  | VPN Server 10.10.10.1 221.158.160.253 |
| --- | --- | --- | --- | --- | --- |
| Inside (10.10.101.0/24) |  | VPN Tunnel |  | Remote (10.10.10.0/24) |  |  |

| Component | IP Address | Role |
| --- | --- | --- |
| Local Host | 10.10.101.2 | Ping source (inside network) |
| VPN Client (Akita) | 10.10.101.1 | Firewall under test |
| VPN Server | 10.10.10.1 | Ping destination (remote network) |

Note: ICMP packets from Local Host (10.10.101.2) to VPN Server (10.10.10.1) traverse the Akita device as forwarded traffic from the inside zone. Therefore, the relevant firewall chain is forward/inside.

4. Test Procedure

Step 0: Configure Route on Local Host

On the Local Host (10.10.101.2), add a static route so that traffic to the remote VPN network (10.10.10.0/24) is forwarded through the Akita VPN Client (10.10.101.1).

Command (on Local Host):

$ sudo ip route add 10.10.10.0/24 via 10.10.101.1

Verify route:

$ ip route show 10.10.10.0/24

Expected Output:

10.10.10.0/24 via 10.10.101.1 dev eth0

Note: Without this route, the Local Host has no path to the 10.10.10.0/24 network and ping will fail regardless of firewall rules.

Step 1: Verify Initial State (No ICMP Forward Rules)

Confirm that no ICMP rules are configured in the forward/inside chain.

Command:

>> firewall show icmp forward inside

Expected Output:

----- Firewall ICMP Rules (forward/inside) -----

(no rules configured)

Note: With no forward ICMP rules, default policy applies. Ping may or may not work depending on the default chain policy. This step confirms a clean starting state.

Step 2: Add ACCEPT Rule for ICMP Echo-Request

Add a rule that accepts ICMP echo-request (ping) packets from the inside zone, then save to apply.

Commands:

>> firewall icmp add forward accept echo-request inside 10.10.101.0/24 allow-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify rule is applied:

>> firewall show icmp forward inside

Expected Output:

| #V4 | RULE NAME | ZONE | SOURCE | TYPE | ACTION | PKTS | BYTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | allow-ping | inside | 10.10.101.0/24 | echo-request | ACCEPT | 0 | 0 |

Step 3: Verify Ping Works (ACCEPT Active)

From Local Host (10.10.101.2), send ping to VPN Server (10.10.10.1).

Command (on Local Host):

$ ping -c 3 10.10.10.1

Expected Output:

PING 10.10.10.1 (10.10.10.1) 56(84) bytes of data.

64 bytes from 10.10.10.1: icmp_seq=1 ttl=63 time=12.3 ms

64 bytes from 10.10.10.1: icmp_seq=2 ttl=63 time=11.8 ms

64 bytes from 10.10.10.1: icmp_seq=3 ttl=63 time=12.1 ms

--- 10.10.10.1 ping statistics ---

3 packets transmitted, 3 received, 0% packet loss

| Expected Result | Ping succeeds — 3 packets transmitted, 3 received, 0% packet loss |
| --- | --- |
| Reason | The allow-ping rule at position 1 accepts ICMP echo-request from 10.10.101.0/24. |

Step 4: Add DROP Rule for ICMP Echo-Request

Add a second rule that drops ICMP echo-request packets. This rule will be placed after the existing ACCEPT rule (position 2).

Commands:

>> firewall icmp add forward drop echo-request inside 10.10.101.0/24 drop-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify both rules:

>> firewall show icmp forward inside

Expected Rule Order:

| #V4 | RULE NAME | ZONE | SOURCE | TYPE | ACTION | PKTS | BYTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | allow-ping | inside | 10.10.101.0/24 | echo-request | ACCEPT | 0 | 0 |
| 2 | drop-ping | inside | 10.10.101.0/24 | echo-request | DROP | 0 | 0 |

Step 5: Verify Ping Still Works (ACCEPT Before DROP)

Ping should still succeed because the ACCEPT rule (position 1) is evaluated before the DROP rule (position 2).

Command (on Local Host):

$ ping -c 3 10.10.10.1

| Expected Result | Ping succeeds — 3 packets transmitted, 3 received, 0% packet loss |
| --- | --- |
| Reason | Rules are evaluated top-down. The ACCEPT rule at position 1 matches first, so the DROP rule at position 2 is never reached. |

Step 6: Move DROP Rule to Position 1 (Before ACCEPT)

Use the firewall icmp move command to reorder the DROP rule from position 2 to position 1, placing it before the ACCEPT rule.

Command Syntax:

firewall icmp move <hook> <zone> <stack> <from> <to>

Commands:

>> firewall icmp move forward inside v4 2 1

>> firewall save

Confirm applied firewall rules ? [Y/n]: Y

Expected Output:

Staged move of forward/inside ICMP rule (v4) from position 2 to 1.

Firewall rules confirmed and saved (IPv4 + IPv6).

Verify new rule order:

>> firewall show icmp forward inside

Expected Rule Order (after move):

| #V4 | RULE NAME | ZONE | SOURCE | TYPE | ACTION | PKTS | BYTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | drop-ping | inside | 10.10.101.0/24 | echo-request | DROP | 0 | 0 |
| 2 | allow-ping | inside | 10.10.101.0/24 | echo-request | ACCEPT | 0 | 0 |

Note: The DROP rule is now at position 1. Since rules are evaluated top-down, it will match before the ACCEPT rule at position 2.

Step 7: Verify Ping Fails (DROP Before ACCEPT)

Ping should now fail because the DROP rule (position 1) is evaluated before the ACCEPT rule (position 2).

Command (on Local Host):

$ ping -c 3 -W 3 10.10.10.1

Expected Output:

PING 10.10.10.1 (10.10.10.1) 56(84) bytes of data.

--- 10.10.10.1 ping statistics ---

3 packets transmitted, 0 received, 100% packet loss

| Expected Result | Ping fails — 3 packets transmitted, 0 received, 100% packet loss |
| --- | --- |
| Reason | The DROP rule at position 1 matches first and silently discards the ICMP echo-request packets. The ACCEPT rule at position 2 is never evaluated. |

5. Cleanup

Remove the test rules and route to restore the original state.

Firewall cleanup (on Akita):

>> firewall icmp del forward drop-ping

>> firewall icmp del forward allow-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify firewall cleanup:

>> firewall show icmp forward inside

Expected Output:

----- Firewall ICMP Rules (forward/inside) -----

(no rules configured)

Route cleanup (on Local Host):

$ sudo ip route del 10.10.10.0/24 via 10.10.101.1

6. Test Summary

| Step | Action | Expected Ping | Result |
| --- | --- | --- | --- |
| 0 | Configure route on Local Host | — |  |
| 1 | Initial state (no rules) | — |  |
| 2 | Add ACCEPT rule (position 1) | — |  |
| 3 | Ping test | ✅ Success | □ Pass / □ Fail |
| 4 | Add DROP rule (position 2) | — |  |
| 5 | Ping test (ACCEPT before DROP) | ✅ Success | □ Pass / □ Fail |
| 6 | Move DROP to position 1 | — |  |
| 7 | Ping test (DROP before ACCEPT) | ❌ Fail | □ Pass / □ Fail |

This test confirms that the firewall icmp move command correctly reorders rules and that the firewall evaluates rules in position order (top-down, first match wins).