Firewall ICMP Rule Priority Test

firewall icmp move Command Verification  |  2026-04-03

Network Topology

| Local Host 10.10.101.2 | → | Akita (VPN Client) 10.10.101.1 | → | VPN Server 10.10.10.1 |
| --- | --- | --- | --- | --- |

Local Host → Akita (forward/inside chain) → VPN tunnel → VPN Server

Prerequisites

- VPN tunnel established (Akita ↔ VPN Server)

- Admin access to Akita CLI (scli)

- Route on Local Host: 10.10.10.0/24 via 10.10.101.1

Test Steps

Step 0: Route Setup (Local Host)

$ sudo ip route add 10.10.10.0/24 via 10.10.101.1

$ ip route show 10.10.10.0/24

10.10.10.0/24 via 10.10.101.1 dev eth0

Step 1: Verify No Rules

>> firewall show icmp forward inside

----- Firewall ICMP Rules (forward/inside) -----

(no rules configured)

Step 2: Add ACCEPT Rule

>> firewall icmp add forward accept echo-request inside 10.10.101.0/24 allow-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show icmp forward inside

#V4  RULE NAME   ZONE    SOURCE          TYPE          ACTION  PKTS  BYTES

1    allow-ping  inside  10.10.101.0/24  echo-request  ACCEPT  0     0

Step 3: Ping Test — Should Succeed

$ ping -c 3 10.10.10.1

✅  3 transmitted, 3 received, 0% loss (ACCEPT at #1 matches)

Step 4: Add DROP Rule

>> firewall icmp add forward drop echo-request inside 10.10.101.0/24 drop-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show icmp forward inside

#V4  RULE NAME   ZONE    SOURCE          TYPE          ACTION  PKTS  BYTES

1    allow-ping  inside  10.10.101.0/24  echo-request  ACCEPT  0     0

2    drop-ping   inside  10.10.101.0/24  echo-request  DROP    0     0

Step 5: Ping Test — Should Still Succeed

$ ping -c 3 10.10.10.1

✅  3 transmitted, 3 received, 0% loss (ACCEPT #1 evaluated before DROP #2)

Step 6: Move DROP to Position 1

>> firewall icmp move forward inside v4 2 1

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show icmp forward inside

#V4  RULE NAME   ZONE    SOURCE          TYPE          ACTION  PKTS  BYTES

1    drop-ping   inside  10.10.101.0/24  echo-request  DROP    0     0

2    allow-ping  inside  10.10.101.0/24  echo-request  ACCEPT  0     0

Step 7: Ping Test — Should Fail

$ ping -c 3 -W 3 10.10.10.1

❌  3 transmitted, 0 received, 100% loss (DROP #1 evaluated before ACCEPT #2)

Cleanup

Akita:

>> firewall icmp del forward drop-ping

>> firewall icmp del forward allow-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Local Host:

$ sudo ip route del 10.10.10.0/24 via 10.10.101.1