Firewall Rule Priority Test Procedure

firewall icmp move / access-policy move — Quick Reference  |  2026-04-06

Network Topology

| Local Host 10.10.101.2 | → | Akita (VPN Client) 10.10.101.1 | → | VPN Server 10.10.10.1 |
| --- | --- | --- | --- | --- |

Local Host → Akita (forward/inside chain) → VPN tunnel → VPN Server

Prerequisites

- VPN tunnel established (Akita ↔ VPN Server)

- Admin access to Akita CLI (scli)

- Route on Local Host: 10.10.10.0/24 via 10.10.101.1

Step 0: Route Setup (Local Host)

$ sudo ip route add 10.10.10.0/24 via 10.10.101.1

$ ip route show 10.10.10.0/24

10.10.10.0/24 via 10.10.101.1 dev eth0

Part A: ICMP Rule Priority (firewall icmp move)

Step 1: Verify No ICMP Rules

>> firewall show icmp forward inside

----- Firewall ICMP Rules (forward/inside) -----

(no rules configured)

Step 2: Add ACCEPT Rule

>> firewall icmp add forward accept echo-request inside 10.10.101.0/24 allow-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show icmp forward inside

#V4  RULE NAME   ZONE    SOURCE          TYPE          ACTION  PKTS  BYTES

1    allow-ping  inside  10.10.101.0/24  echo-request  ACCEPT  0     0

Step 3: Ping Test — Should Succeed (on Local Host)

$ ping -c 3 10.10.10.1

✅  3 transmitted, 3 received, 0% loss (ACCEPT at #1 matches)

Step 4: Add DROP Rule

>> firewall icmp add forward drop echo-request inside 10.10.101.0/24 drop-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show icmp forward inside

#V4  RULE NAME   ZONE    SOURCE          TYPE          ACTION  PKTS  BYTES

1    allow-ping  inside  10.10.101.0/24  echo-request  ACCEPT  0     0

2    drop-ping   inside  10.10.101.0/24  echo-request  DROP    0     0

Step 5: Ping Test — Should Still Succeed (on Local Host)

$ ping -c 3 10.10.10.1

✅  3 transmitted, 3 received, 0% loss (ACCEPT #1 evaluated before DROP #2)

Step 6: Move DROP to Position 1

>> firewall icmp move forward inside v4 2 1

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Staged move of forward/inside ICMP rule (v4) from position 2 to 1.

Firewall rules confirmed and saved (IPv4 + IPv6).

Verify:

>> firewall show icmp forward inside

#V4  RULE NAME   ZONE    SOURCE          TYPE          ACTION  PKTS  BYTES

1    drop-ping   inside  10.10.101.0/24  echo-request  DROP    0     0

2    allow-ping  inside  10.10.101.0/24  echo-request  ACCEPT  0     0

Step 7: Ping Test — Should Fail (on Local Host)

$ ping -c 3 -W 3 10.10.10.1

❌  3 transmitted, 0 received, 100% loss (DROP #1 evaluated before ACCEPT #2)

Step 8: Cleanup

>> firewall icmp del forward drop-ping

>> firewall icmp del forward allow-ping

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show icmp forward inside

(no rules configured)

Part B: Access-Policy Rule Priority (access-policy move)

Access-policy rules filter on protocol/port/address. We use -p icmp to control ping traffic, verifying the same move behavior as Part A.

Step 1: Verify No Access-Policy Rules

>> firewall show access-policy inside

----- Access Policy Rules (inside) -----

(no rules configured)

Step 2: Add ACCEPT Rule

>> firewall access-policy add inside accept allow-ping-ap 10.10.101.0/24 10.10.10.0/24 -p icmp

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show access-policy inside

#V4  RULE NAME      SOURCE          DEST           PROTO  SPORT  DPORT  ACTION  PKTS  BYTES

1    allow-ping-ap  10.10.101.0/24  10.10.10.0/24  icmp   -      -      ACCEPT  0     0

Step 3: Ping Test — Should Succeed (on Local Host)

$ ping -c 3 10.10.10.1

✅  3 transmitted, 3 received, 0% loss (ACCEPT at #1 matches)

Step 4: Add DROP Rule

>> firewall access-policy add inside drop drop-ping-ap 10.10.101.0/24 10.10.10.0/24 -p icmp

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show access-policy inside

#V4  RULE NAME      SOURCE          DEST           PROTO  SPORT  DPORT  ACTION  PKTS  BYTES

1    allow-ping-ap  10.10.101.0/24  10.10.10.0/24  icmp   -      -      ACCEPT  0     0

2    drop-ping-ap   10.10.101.0/24  10.10.10.0/24  icmp   -      -      DROP    0     0

Step 5: Ping Test — Should Still Succeed (on Local Host)

$ ping -c 3 10.10.10.1

✅  3 transmitted, 3 received, 0% loss (ACCEPT #1 evaluated before DROP #2)

Step 6: Move DROP to Position 1

>> firewall access-policy move inside v4 2 1

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Staged move of inside access-policy rule (v4) from position 2 to 1.

Firewall rules confirmed and saved (IPv4 + IPv6).

Verify:

>> firewall show access-policy inside

#V4  RULE NAME     SOURCE          DEST           PROTO  SPORT  DPORT  ACTION  PKTS  BYTES

1    drop-ping-ap   10.10.101.0/24  10.10.10.0/24  icmp   -      -      DROP    0     0

2    allow-ping-ap  10.10.101.0/24  10.10.10.0/24  icmp   -      -      ACCEPT  0     0

Step 7: Ping Test — Should Fail (on Local Host)

$ ping -c 3 -W 3 10.10.10.1

❌  3 transmitted, 0 received, 100% loss (DROP #1 evaluated before ACCEPT #2)

Step 8: Cleanup

>> firewall access-policy del inside drop-ping-ap

>> firewall access-policy del inside allow-ping-ap

>> firewall save

Confirm applied firewall rules? [Y/n]: Y

Verify:

>> firewall show access-policy inside

(no rules configured)

Final Cleanup: Route

Local Host:

$ sudo ip route del 10.10.10.0/24 via 10.10.101.1